# Design Specification: Custom Meshtastic LoRa PCB

​Project: ESP32 + RA-01SH Integration with Native USB-C Charging & Data

Date: January 11, 2026

Version: 1.0

## ​1. Project Overview

​The objective is to design a production-ready custom PCB for Meshtastic LoRa networking. The design integrates an ESP32-WROOM-32E (MCU) and an Ai-Thinker RA-01SH (LoRa) module.

​Critical Requirement: The board must feature "native" USB-C support. This includes onboard Li-Ion battery charging and a USB-to-UART bridge for direct firmware flashing and debugging via the USB-C port. No external modules (like TP4056 boards) are to be used; all circuits must be integrated directly onto the main PCB.

## ​2. Component Selection (Bill of Materials)

​Use the following specific components (or functional equivalents) for the design:

- ​MCU: ESP32-WROOM-32E (Non-USB version).

- ​LoRa Module: Ai-Thinker RA-01SH (SX1262).

- ​USB-UART Bridge: CH340C (SOIC-16 package preferred; requires no external crystal).

- ​Battery Charger: MCP73831 (SOT-23-5); Configured for ~500mA charge rate.

- ​Voltage Regulator (LDO): AP2112K-3.3 (SOT-23-5); Low Dropout, 600mA output.

- ​Battery Protection: DW01A + FS8205A (Standard Li-Ion protection circuit).

- ​Connector: USB Type-C Receptacle (16-pin or 6-pin simplified 2.0).

## ​3. Schematic Design Requirements

### ​A. Critical Engineering Corrections (MUST IMPLEMENT)

​The following supersede any prior generated drafts:

- ​USB Data Resistors: Use 22Ω series resistors on D+ and D- lines. (Do NOT use "222" / 2.2kΩ).

- ​SPI Resistors: Use 33Ω series resistors on SCK, MISO, and MOSI lines. (Do NOT use "332" / 3.3kΩ).

- ​GPIO0 Configuration: This pin must be Pulled UP (10kΩ) to 3.3V for normal boot. It is only pulled low by the Auto-Reset circuit during flashing.

- ​LoRa Power: A 100µF bulk capacitor is mandatory near the RA-01SH VCC pin to prevent brownouts during transmission.

### ​B. Sub-Circuit Logic

​1. Power & Charging Pathway

- ​USB-C Config: Connect 5.1kΩ pull-down resistors to GND on pins CC1 and CC2.

- ​Charger (MCP73831):

  - ​PROG Resistor: 2kΩ (Sets charging current to 500mA).

  - ​Input/Output Caps: 10µF ceramic.

.

- ​Regulator (AP2112K): Input from Battery (VBAT_SYS); Output to 3.3V rail.

​2. USB-to-UART & Auto-Reset

- ​Bridge: CH340C.

- ​Auto-Reset Circuit (Required for Flashing): Implement the standard "Two-Transistor" logic using S8050 (NPN) transistors.

  - ​Transistor Q1 (Reset): Base controlled by RTS; Collector pulls ESP32 EN to GND.

  - ​Transistor Q2 (Boot): Base controlled by DTR; Collector pulls ESP32 IO0 to GND.

- ​Timing: Place a 1µF capacitor between ESP32 EN pin and GND to ensure stable reset timing.

​3. Pin Mapping (Netlist)

| Signal Name | Source (ESP32) | Target (RA-01SH) |
| --- | --- | --- |
| NSS (CS) | GPIO 5 | NSS |
| SCK | GPIO 18 | SCK |
| MISO | GPIO 19 | MISO |
| MOSI | GPIO 23 | MOSI |
| DIO1 | GPIO 26 | DIO1 |
| RESET | GPIO 14 | RESET |
| BUSY | GPIO 33 | BUSY |

## 4. PCB Layout Guidelines

### ​A. RF Design (LoRa Section)

- ​Antenna Trace: Route the trace from RA-01SH "ANT" pin to the SMA/IPEX connector using a calculated 50Ω impedance.

- ​Trace Geometry: Keep this trace short, straight, and on the Top Layer.

- ​Ground Reference: Ensure a solid, unbroken Ground Plane exists on the layer immediately below the RF trace.

- ​Stitching: Add "shielding vias" (stitching vias) along both sides of the RF trace and around the RA-01SH module.

### ​B. Component Placement

- ​ESP32-WROOM: Place at the board edge. The PCB antenna area must hang over the edge or have a strict Keep-Out Zone (No copper on any layer) underneath it.

- ​Isolation: Place the RA-01SH module on the opposite side of the board from the ESP32 antenna to minimize WiFi/LoRa interference.

- ​Power: Place the LDO and Battery Charger close to the USB connector.

### ​C. Power Routing

- ​Trace Widths:

  - ​Power (VBUS, VBAT, 3V3, GND): Minimum 40 mil (1.0mm).

  - ​Signal (GPIO, SPI, UART): Minimum 10 mil (0.25mm).

.

- ​Star Routing: Route the 3.3V line to the RA-01SH directly from the LDO output capacitor (do not daisy-chain through the ESP32).

## ​5. Manufacturing Specifications (DRC)

​Please ensure the design passes the following Design Rules Check (DRC) for standard 2-layer FR4 manufacturing:

- ​Min Track Width: 6 mil (0.15mm)

- ​Min Clearance: 6 mil (0.15mm)

- ​Min Via Size: 0.3mm Hole / 0.6mm Diameter

- ​Copper Weight: 1 oz

- ​PCB Thickness: 1.6mm

## ​6. Deliverables

- ​Gerber Files: (RS-274X format) including all Copper, Solder Mask, Silkscreen, Edge Cuts, and Drill files (Excellon).

- ​Assembly Files: BOM (CSV) and CPL/Pick-and-Place (CSV) for PCBA.

- ​Source Files: Native project files (KiCad/EasyEDA/Altium) and PDF Schematic.